The buck-boost converter module has two submodules which is referred to LCD module and voltage converter module.

Components used in LCD module:

- HK32F030F4P6 – 1

- Manufacturer Shenxhen Hangshun Chip Technology

- Chinese datasheet converted to English version using:

  - https://www.onlinedoctranslator.com

- Operating range: 2.0V ~ 5.5V

- Clock: upto 72 MHz

- Reset; software reset, power on reset, external pin reset, watchdog timer reset, power mode reset, low voltage detection,

- 64 KBytes memory

- 16 ADC

- On board temperature sensor

- 7 Timer

- 2 I2C, USART, SPI

- HT1621

- HOLTEK

- RAM Mapping 32x4 LCD Controller for i/o MCU

- Operating range: 2.4V ~ 5.2V

- Built in 256 KHz RC Oscilator

- Can be configured as ½ or 1/3 bias can be software configured

Components used in converter submodule:

- Lm358 - 2

- FP5139

  - Boost conver control ic

  - Operating voltage range 1.8-15v

- SS8050 NPN transistor

  - Collector current: 1.5A

- SS8550 PNP transistor

  - Collector current: -1.5A

- Special Resistor:

  - R025: 0.025Ohm

  - R202: 2K Ohm

- ME6209: voltage regulator: 3.3v regulator

  - Operating range: max 18v, max output current 250mA

- RU7570L

  - 75V 70A N Channel Power MOSFET

- SS56

  - Schottky barrier rectifiers

  - Reveres voltage 60 volts

  - Forward current 5.0 amps

- MST5350B

  - Voltage regulator

  - Input voltage up to 35V

  - Output current <200mA

- Choke

  - 4 pin SMT component

- 31 resistors + 7 open circuit(High resistance) = 38 resistor

- 21 SMT Capacitors

- THT capacitors

  - 220uf

  - 2 35V – 470uF – 2

  - 35V 100uF -1

- Screw terminals

  - 4x1 screw terminal